Exp No: 2

GENERATION OF SQUARE WAVE USING PROTEUS

OBSERVATION:

- The square wave generator circuit was successfully designed and simulated in Proteus using the required components.

- The output waveform observed on the oscilloscope was a continuous square wave with equal HIGH and LOW time periods.

- The frequency and amplitude of the generated square wave were verified by changing the circuit parameters and observing the output waveform in Proteus.

PROGRAM

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT