Exp No: 3

STEPPER MOTOR USING AT89C51 USING PROTEUS

OBSERVATION:

- The AT89C51 microcontroller successfully controls the stepper motor by sending the correct sequence of pulses to the motor driver, causing the motor to rotate.

- The stepper motor rotates smoothly in the programmed direction (clockwise or anticlockwise) with the speed depending on the delay provided in the program.

- The Proteus simulation verifies proper interfacing between the AT89C51, motor driver (ULN2003/L293D), and stepper motor, showing accurate step-by-step motor movement without errors.

PROGRAM

ORG 0000H

UP: MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP UP

DELAY:MOV R4,#18

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT